📘 AIoT-Based Cold Supply Chain Monitoring System

(ESP32 Firmware + ML Anomaly Detection + Flutter Mobile App)

📌 Abstract

Cold supply chains require continuous supervision to maintain the quality of perishable goods such as fruits, vegetables, dairy, meat, and pharmaceuticals.
This project implements an AIoT-driven cold chain monitoring system that measures key environmental parameters using an ESP32 microcontroller, detects anomalies using an Isolation Forest machine learning model, and visualizes real-time data in a Flutter mobile application.

The solution provides a complete pipeline:
✔ Real-time sensor data collection
✔ Cloud data upload using ThingSpeak
✔ Machine learning anomaly detection
✔ User interface for monitoring + alerting

1️⃣ IoT Hardware + ESP32 Firmware

1.1 Hardware Components Used

| Component | Purpose |
| --- | --- |
| ESP32 Dev Board | Main microcontroller for data reading & WiFi transmission |
| BMP180 / BMP280 Sensor | Measures temperature & pressure |
| Ultrasonic Sensor (HC-SR04) | Measures container fill-level (distance) |
| Capacitive Moisture Sensor | Detects leakage / moisture inside container |
| WiFi / ThingSpeak Cloud | Used for real-time data upload |

1.2 Sensor Parameters Collected

The ESP32 collects 5 key metrics:

| Field | Sensor | Use Case |
| --- | --- | --- |
| temperature | BMP sensor | Detect spoilage / temperature breach |
| pressure | BMP sensor | Atmospheric changes / refrigeration health |
| distance | Ultrasonic | Filling level, sudden drop alerts (door open/theft) |
| moisture | Capacitive moisture | Leakage detection |
| spoilageRisk | Calculated field | Quick risk estimation (threshold-based) |

1.3 ESP32 Firmware Workflow

- Initialize sensors (BMP, moisture, ultrasonic)

- Read sensor values periodically

- Compute spoilage risk score

- Send data to ThingSpeak using HTTP GET request

- Repeat after fixed interval

1.4 ESP32 Sample Code Logic (Simplified)

float temp = bmp.readTemperature();

float pressure = bmp.readPressure() / 100.0;

float distance = getDistanceCM();

int moisture = getMoisture();

int spoilageRisk = (temp > 8 || moisture > 50) ? 90 : 20;

String url = server +

"&field1=" + String(temp) +

"&field2=" + String(pressure) +

"&field3=" + String(distance) +

"&field4=" + String(moisture) +

"&field5=" + String(spoilageRisk);

✔ If temp > 8°C → spoilage risk increases
✔ If moisture > 50% → leakage alert

2️⃣ Machine Learning: Isolation Forest Anomaly Detection

The ML model identifies unusual patterns, such as:

- Sudden temperature spike

- Unexpected drop in fill level

- Pressure anomalies

- Continuous moisture presence

- Abnormal spoilage risk behaviour

2.1 Dataset and Features

Dataset consists of historical sensor data:

| Feature | Purpose |
| --- | --- |
| temperature | main spoilage parameter |
| pressure | indicates cooling system health |
| distance | fill-level & door activity |
| moisture | leakage detection |
| spoilageRisk | derived indicator |

Feature Order:

["temperature", "pressure", "distance", "moisture", "spoilageRisk"]

2.2 Isolation Forest Model

Isolation Forest is ideal for anomaly detection because:

- Works with small datasets

- Detects unusual behaviour

- Fast prediction

- No labeling required

Output of model:

| Value | Meaning |
| --- | --- |
| 1 | Normal reading |
| -1 | Anomaly detected |
| score | How normal/abnormal the sample is |

2.3 Model Serving using FastAPI

FastAPI Route Logic

/predict accepts JSON readings:

@app.post("/predict")

def predict(readings: List[Reading]):

df = pd.DataFrame([r.model_dump() for r in readings])

X = df.reindex(columns=FEATURE_ORDER).astype(float).values

preds = model.predict(X)

scores = model.decision_function(X)

return [{"prediction": int(p), "score": float(s)} for p, s in zip(preds, scores)]

✔ Converts input → DataFrame
✔ Ensures feature order
✔ Runs Isolation Forest
✔ Returns anomaly result

2.4 API Endpoints

| Endpoint | Description |
| --- | --- |
| GET /health | Check API status |
| POST /predict | Send sensor readings & get anomaly results |

Example request to /predict:

[

{

"temperature": 25.3,

"pressure": 987,

"distance": 17,

"moisture": 40,

"spoilageRisk": 20

}

]

3️⃣ Flutter Mobile/Web Application

The Flutter application provides:

✔ Live dashboard of all sensor readings
✔ Graphs of historical data
✔ Anomaly alerts via ML API
✔ Notifications & UI banners
✔ Tabbed interface for navigation

3.1 Flutter Modules

| File | Purpose |
| --- | --- |
| cold_chain_api.dart | Fetches data from ThingSpeak |
| anomaly_service.dart | Sends sensor data to FastAPI |
| dashboard_screen.dart | Shows real-time UI, alerts, charts |
| notification_service.dart | Push/local alerts |
| main.dart | App entry point |

3.2 Calling FastAPI from Flutter

final anomaly = AnomalyService(baseUrl: "http://127.0.0.1:5000");

final response = await anomaly.predictSingle({

"temperature": temp,

"pressure": pressure,

"distance": distance,

"moisture": moisture,

"spoilageRisk": spoilageRisk

});

If prediction is -1, Flutter shows a red banner and triggers a notification.

3.3 Dashboard Features

- Sensor cards (temperature, moisture, pressure, distance)

- Live updating UI

- Anomaly indicator

- History graph using fl_chart

- Pull-to-refresh

4️⃣ System Architecture

┌─────────────┐      WiFi       ┌──────────────┐

│   ESP32      │  ───────────▶   │ ThingSpeak     │

│ Sensors:     │                │ Cloud Storage   │

│ Temp, Moist  │                └──────┬─────────┘

│ Distance     │                       │ API Fetch

└──────┬──────┘                       ▼

│                        ┌──────────────┐

│                        │ Flutter App   │

└── Sensor Data ───────▶ │ Dashboard UI  │

│ Calls FastAPI │

└──────┬────────┘

│

ML Prediction

│

┌─────────▼─────────┐

│  FastAPI Server    │

│ Isolation Forest   │

└────────────────────┘

5️⃣ How to Run Each Component

🟦 Part 1 — Running the ESP32 Firmware

Requirements:

- ESP32 board

- Arduino IDE / PlatformIO

- WiFi credentials

- ThingSpeak API Key

Steps:

- Install ESP32 board support in Arduino IDE

- Connect sensors correctly

- Insert WiFi SSID & password

- Insert ThingSpeak Write API key

- Upload code to ESP32

- Open Serial Monitor → verify data sending

🟩 Part 2 — Running the FastAPI ML Server

Requirements:

pip install fastapi uvicorn pandas joblib scikit-learn

Start the server:

uvicorn app:app --host 0.0.0.0 --port 5000 --reload

Test:

http://127.0.0.1:5000/health

Predict:

Use Postman or PowerShell:

Invoke-RestMethod -Uri "http://127.0.0.1:5000/predict" -Method POST -ContentType "application/json" -Body '[{"temperature":25,"pressure":990,"distance":20,"moisture":40,"spoilageRisk":30}]'

🟨 Part 3 — Running the Flutter App

Install dependencies:

flutter pub get

Run on device:

flutter run

Things to configure in Flutter:

- Base URL of API

  - Web → "http://127.0.0.1:5000"

  - Android Emulator → "http://10.0.2.2:5000"

- ThingSpeak Channel ID & Read API Key

6️⃣ Conclusion

This project demonstrates a complete end-to-end AIoT solution:

- IoT sensors capture real-time environmental data

- Machine learning model identifies anomalies

- FastAPI provides a reliable prediction backend

- Flutter UI displays live readings and alerts users

Such a system can significantly reduce spoilage, detect faults early, and improve operational efficiency in cold supply chains.